ZIMBABWE NATIONAL GEOSPATIAL AND SPACE AGENCY (ZINGSA) In Partnership with: Ministry of Transport & Infrastructural Development Department of Roads

ROAD CONDITION SURVEY SYSTEM

USER MANUAL

Mobile Application & Web Dashboard

| Document Version | v1.0 |
| --- | --- |
| Issue Date | July 2026 |
| Prepared By | ZINGSA — Zimbabwe National Geospatial and Space Agency |
| Classification | INTERNAL USE ONLY |
| Survey Year | 2026 |

Field Telemetry System v2.0 ¦ Survey Year 2026 ¦ Highways: A1 - A2 - A3 - A4 - A5

# TABLE OF CONTENTS

| 1 | Introduction | 3 |
| --- | --- | --- |
| 2 | System Overview & Architecture | 4 |
| 3 | Getting Started | 5 |
| 4 | Mobile Application - Field Surveyor Guide | 6 |
| 4.1 | Installation & First Launch | 6 |
| 4.2 | Home Screen & Navigation | 7 |
| 4.3 | Creating a New Survey | 8 |
| 4.4 | Asset Categories | 9 |
| 4.5 | Survey Form - Common Fields | 11 |
| 4.6 | Asset-Specific Survey Forms | 12 |
| 4.7 | GPS Coordinate Capture | 20 |
| 4.8 | Photo Capture | 21 |
| 4.9 | Draft Queue & Offline Storage | 22 |
| 4.10 | Synchronising Data to Server | 23 |
| 4.11 | Settings & Configuration | 24 |
| 5 | Web Dashboard - Administrator Guide | 25 |
| 5.1 | Accessing the Dashboard | 25 |
| 5.2 | Layout & Navigation | 26 |
| 5.3 | Map View | 27 |
| 5.4 | Assets Panel | 28 |
| 5.5 | Dashboard Module | 29 |
| 5.6 | Highways Module | 29 |
| 5.7 | Analytics Module | 30 |
| 5.8 | Survey Module | 31 |
| 5.9 | Database Module | 31 |
| 5.10 | Export Module | 32 |
| 5.11 | Right-Side Analytics Panel | 33 |
| 5.12 | Refreshing Live Data | 33 |
| 6 | Data Synchronisation & Offline Operation | 34 |
| 7 | System Administration | 35 |
| 8 | Troubleshooting | 36 |
| 9 | Glossary | 38 |
| Appendix A | Central Database Integration | 40 |
| Appendix B | API Endpoint Reference | 41 |

# 1.  INTRODUCTION

The Zimbabwe National Geospatial and Space Agency (ZINGSA), in partnership with the Ministry of Transport and Infrastructural Development (MOTID) - Department of Roads, has deployed the Road Condition Survey System to support the national mandate of monitoring, cataloguing, and maintaining Zimbabwe's road network.

This system consists of two integrated components:

Mobile Application (Android) - Used by field surveyors to collect road asset condition data in the field, including in areas with no internet connectivity.

Web Dashboard - Used by Roads Department administrators and engineers to view, analyse, and export the collected survey data via an interactive map and reporting interface.

1.1  Purpose of this Document

This User Manual provides step-by-step instructions for both field surveyors operating the Mobile Application and administrators using the Web Dashboard. It covers all features, forms, and workflows required to operate the system effectively.

1.2  Scope

This manual applies to:

Field surveyors assigned to road condition survey teams

Roads Department administrators and engineers

ZINGSA GIS and data management staff

ICT staff responsible for system maintenance

1.3  Document Conventions

| Convention | Meaning |
| --- | --- |
| Bold text | Buttons, menu items, and field labels in the application |
| Italic text | Values to be entered or selected |
| [Icon label] | Reference to an icon button on screen |
| NOTE: | Important information the user should be aware of |
| WARNING: | Action that may result in data loss if not followed correctly |

# 2.  SYSTEM OVERVIEW & ARCHITECTURE

The Road Condition Survey System is a full-stack field telemetry platform developed and operated by ZINGSA in support of the MOTID Department of Roads. It comprises three tiers:

| Tier | Technology | Purpose |
| --- | --- | --- |
| Mobile App (Android) | Capacitor + React (TypeScript) | Field data collection, offline storage, GPS capture, photo attachment, draft queue, one-tap sync |
| Backend API | Python FastAPI | Receives survey submissions from mobile, proxies data from Central Database, caches records, serves REST API to dashboard |
| Web Dashboard | Next.js (React) | Interactive Leaflet map, multi-module analytics, data export, live/offline data fallback |

2.1  Data Flow

Field Surveyor (Mobile App) --> SYNC --> FastAPI Backend --> FETCH --> Web Dashboard

2.2  Covered Road Asset Categories

| Category | Asset Types |
| --- | --- |
| Roads | Sealed Roads, Gravel Roads, Earth Roads |
| Structures | Bridges, Footbridges, Rail Crossings, Tollgates |
| Amenities | Lay-bys, Bus Stops, Junctions, Road Signs |
| Drainage | Shelvets, Culverts, Piped Causeways, Drifts, Grids |
| Traffic | Traffic Lights, Streetlights |

# 3.  GETTING STARTED

3.1  System Requirements

| Component | Requirement | Notes |
| --- | --- | --- |
| Mobile App | Android 8.0 (Oreo) or higher | 1 GB RAM minimum; camera & GPS permissions required |
| Web Dashboard | Any modern web browser (Chrome, Edge, Firefox) | Internet connection recommended; works with cached data offline |
| Backend Server | Python 3.9+, FastAPI | Deployed on-premises or cloud server |

3.2  Obtaining the Mobile Application

Step 1: Obtain the file roads.apk from your ZINGSA supervisor or ICT department.

Step 2: On your Android device, go to Settings > Security > Enable 'Install from Unknown Sources'.

Step 3: Open the APK file using a file manager and tap Install.

Step 4: Grant all requested permissions - Camera, Location (GPS), and Storage.

Step 5: Launch the app. The app icon will appear on your home screen.

NOTE The application does not require a Google Play Store account or internet connection to install. It is a self-contained enterprise APK distributed by ZINGSA.

# 4.  MOBILE APPLICATION - FIELD SURVEYOR GUIDE

The Mobile Application is the primary data collection tool for field surveyors. It is designed to work in offline environments and syncs data when a network connection becomes available.

## 4.1  Installation & First Launch

Tap Allow when prompted for GPS/Location permissions.

Tap Allow when prompted for Camera permissions.

Navigate to the Settings tab and enter your Surveyor Name and the Server URL provided by your supervisor.

The app immediately begins warming up the GPS in the background to improve accuracy.

## 4.2  Home Screen & Navigation

| Tab | Description |
| --- | --- |
| Home | Welcome screen showing system status and quick-start guide |
| New Survey | Start a new road asset survey - select asset category and fill in the form |
| Queue | View and manage saved drafts and queued submissions pending sync |
| Settings | Configure server URL, default surveyor name, and GPS accuracy settings |

Figure: Mobile Application Home Screen

## 4.3  Creating a New Survey

Step 1: Tap the New Survey tab.

Step 2: Browse the asset category grid (colour-coded by type).

Step 3: Tap the asset category you wish to survey.

Step 4: The survey form for that asset type will open.

Step 5: Fill in all required fields.

Step 6: Capture GPS coordinates and optionally attach a photo.

Step 7: Tap Save Draft to save locally, or Submit to queue for upload.

Figure: New Survey Asset Selection Grid

AUTO-SAVE The app automatically saves your form progress every 400ms as you type. If the app closes unexpectedly, you will be prompted to recover your unsaved work next time you open a form.

## 4.4  Asset Categories

| Asset Type | Category | Description |
| --- | --- | --- |
| Sealed Roads | Road | Paved highways & asphalt roads. Track segment lines & inspect cracks, potholes, rutting. |
| Gravel Roads | Road | Unsealed gravel networks. Monitor corrugations, thickness, potholes & grading. |
| Earth Roads | Road | Natural dirt tracks. Verify seasonal passability, terrain & water ponding points. |
| Bridges | Structure | Major highway bridges. Assess joints, bearing conditions, parapets & structural state. |
| Footbridge | Structure | Pedestrian crossings. Survey materials, deck spans and walkway safety railings. |
| Rail Crossing | Structure | At-grade or separated railway intersections. Inspect signal guards & warning signage. |
| Tollgate | Structure | Plaza collection gates. Check number of lanes, status and control mechanism integrity. |
| Lay-by | Amenity | Roadside rest zones. Check surface type, length and cleanliness state. |
| Bus Stop | Amenity | Passenger bays & shelters. Record shelter structural status and bay surface conditions. |
| Junction | Amenity | Main route junctions. Monitor markings quality, warning signs & directional beacons. |
| Road Sign | Amenity | Speed limits & safety signs. Inspect SADC compliance, poles and reflectivity. |
| Shelvets | Drainage | Masonry or concrete side drainage. Monitor siltation and crack damage. |
| Culverts | Drainage | Concrete pipe/box underpasses. Check blocking debris & structural walls. |
| Piped Causeway | Drainage | Low-level river crossings with pipe channels. Survey serviceability state. |
| Drift | Drainage | Concrete slab splashways. Inspect water flow depths and slab erosion. |
| Grid | Drainage | Vehicle cattle grids. Check frame stability, bar spacing & cleanout pit. |
| Traffic Lights | Traffic | Intersection robot signaling. Check solar/grid power, phases and housing damage. |
| Streetlights | Traffic | Pole lighting networks. Count active LED fixtures, check cables & battery cabinets. |

## 4.5  Survey Form - Common Fields

| Field | Description | Example / Action |
| --- | --- | --- |
| Road Name * | The national highway or road route | A4 Highway (Harare-Masvingo-Beitbridge) |
| Section Name * | Descriptive name for the road section | Masvingo to Beitbridge, km 145 |
| Surveyor Name * | Full name of the field surveyor | John Moyo |
| Survey Date * | Date the survey was conducted | 2026-07-13 |
| GPS Coordinates* | Latitude/Longitude from device GPS | Tap 'Capture GPS' button |
| Photo | Optional photo attachment of the asset | Tap 'Take Photo' button |
| SADC Compliant | Whether the asset meets SADC standards | Yes / No |

## 4.6  Asset-Specific Survey Forms

4.6.1  Sealed Roads

Figure: Sealed Roads Survey Form

| Field | Description | Options |
| --- | --- | --- |
| Road Class | Classification of the road | Primary / Secondary / Tertiary |
| Surface Type | Type of sealed surface | Wide Mat SS / Otta Seal / Double Seal |
| Climate Zone | Climate of the area | Arid / Semi-arid / Moderate / Tropical |
| Terrain | Topographic terrain type | Flat / Rolling / Mountainous |
| Road Authority | Managing authority | RDC / DDF / MOTID Central |
| Length (km) | Segment length in kilometres | e.g. 12.5 |
| Width (m) | Road carriageway width in metres | e.g. 7.0 |
| Drainage Type | Type of road drainage | V-Drain / Kerb & Channel / Table Drain |
| Narrow Cracks | Hairline/narrow crack condition | No Cracks / <5% / 5-15% / >15% |
| Wide Cracks | Wide/alligator crack condition | No Cracks / <5% / 5-15% / >15% |
| Potholes/Patches | Pothole and patching condition | Good / Fair / Poor / Very Poor |
| Rutting | Wheel track rutting severity | No Rutting / <5mm / 5-20mm / >20mm |
| Edge Breaks | Road edge break condition | None / <50mm / 50-100mm / >100mm |
| Drainage Cond. | Overall drainage condition | Good / Fair / Poor / Blocked |
| Ravelling | Surface ravelling extent | None / Low / Medium / High |
| Riding Quality | Overall riding quality | Good / Fair / Poor / Very Poor |
| Road Markings | Presence of road markings | Yes / No |
| Road Studs | Presence of road studs/cats eyes | Yes / No |
| Passability | When road is passable | All Year Round / Wet Season Restricted / Dry Season Only |
| Year Constructed | Year road was built | e.g. 2005 |

4.6.2  Bridges

Figure: Bridges Survey Form

| Field | Description | Options |
| --- | --- | --- |
| Bridge Name | Name or identifier of the bridge | e.g. Runde River Bridge |
| Crossing Over | What the bridge crosses | River / Railway / Road / Valley |
| Bridge Type | Structural type | HLDC / Box Culvert / Timber / Steel Truss |
| Bearing Type | Type of bearing used | Elastomeric / Pot / Roller / Pinned |
| Expansion Joints | Condition of expansion joints | Good / Fair / Poor / Absent |
| Bearings State | Condition of bearings | Good / Fair / Poor / Failed |
| Parapet | Condition of bridge railings/parapets | Undamaged / Minor / Major / Missing |
| Chemical Effect | Evidence of chemical corrosion | None / Minor / Severe |
| Vegetation Growth | Vegetation growing on bridge | No / Yes - Minor / Yes - Severe |
| Drainage | Bridge drainage condition | Good / Blocked / Absent |
| Overall Condition | Overall structural assessment | Good / Fair / Poor / Critical |

4.6.3  Gravel Roads

| Field | Description | Options |
| --- | --- | --- |
| Road Class | Road classification | Urban Collector / Rural Primary / Feeder Road |
| Gravel Thickness | Estimated gravel layer thickness | <100mm / 100-150mm / >150mm |
| Corrugations | Severity of surface corrugations | None / Low / Medium / High |
| Cross Section | Camber/cross-section shape | Flat / Slightly Cambered / Well Cambered |
| Potholes | Pothole density | None / Low / Medium / High |
| Riding Quality | Overall ride quality | Good / Fair / Poor / Very Poor |
| Passability | Seasonal passability | All Year Round / Wet Season Restricted / Dry Season Only |

4.6.4  Culverts & Drainage

| Field | Description | Options |
| --- | --- | --- |
| Culvert Class | Culvert classification | Pipe Culvert / Box Culvert / Multi-Pipe |
| Material | Culvert pipe material | Concrete / Steel / HDPE / Corrugated |
| Serviceability | Overall serviceability state | Good / Fair / Poor / Blocked |
| Shelvet Type | Type of shelvet structure | Armco / Masonry / Concrete |
| Drift Passability | When drift is passable | All Year / Dry Season Only |

4.6.5  Traffic Assets

| Field | Description | Options |
| --- | --- | --- |
| Location | Intersection or street name | e.g. Corner Samora Machel / 4th St |
| Type | Traffic light type | Standard / Solar / Flashing Amber |
| Phases | Number of signal phases | 3 / 4 / 2-phase pedestrian |
| Operational | Currently operational? | Yes / No |
| Condition | Physical condition | Good / Fair / Poor / Vandalized |
| Power Source | Streetlight power supply | Grid / Solar / Battery Hybrid |
| Count | Number of streetlight poles surveyed | e.g. 24 |

## 4.7  GPS Coordinate Capture

Tap the Capture GPS button on the survey form.

The app displays a GPS status indicator showing current accuracy in metres.

Wait until the accuracy is within your configured GPS Accuracy Limit (default: 3 metres).

Once the accuracy threshold is met, coordinates are automatically confirmed and locked.

The Latitude and Longitude fields will be populated automatically.

TIP - GPS Accuracy For best accuracy, stand in an open area away from buildings and tree cover. Allow 30-60 seconds for the GPS signal to stabilise.

## 4.8  Photo Capture

Tap the Take Photo button on the survey form.

The device camera will open.

Frame the asset clearly and capture the image.

Photos are embedded in the draft record and uploaded during sync.

## 4.9  Draft Queue & Offline Storage

| Status | Description | Action |
| --- | --- | --- |
| Draft | Surveys saved but not yet ready for submission. Can be edited. | Tap a draft to open and continue editing |
| Queued | Surveys marked as ready to submit - awaiting network sync. | Will auto-upload when connected |
| Submitted | Surveys successfully uploaded to the server. | Cannot be edited after upload |

Figure: Draft Queue and Sync Management

## 4.10  Synchronising Data to Server

Ensure your device has a WiFi or mobile data connection.

Navigate to the Queue tab.

Tap the Sync All button.

A progress bar displays 'Uploading X of Y records...'

Successfully uploaded records show a green checkmark.

WARNING Do not close or lock the app while a sync is in progress. Wait until all records show a green checkmark before closing the app.

## 4.11  Settings & Configuration

| Setting | Description | Example |
| --- | --- | --- |
| Server URL | The URL of the backend API server | http://192.168.1.10:3002 |
| Default Surveyor | Your name, pre-filled in new survey forms | e.g. Tendai Chikwanda |
| GPS Accuracy Limit | Maximum acceptable GPS error in metres | Default: 3.0m |
| Test Connection | Tap to verify the app can reach the server | Shows Online or Offline |

Figure: Application Configuration Settings

# 5.  WEB DASHBOARD - ADMINISTRATOR GUIDE

The Web Dashboard is a browser-based application used by Roads Department administrators, ZINGSA analysts, and engineers to view, query, and export road condition survey data.

## 5.1  Accessing the Dashboard

Open Google Chrome, Microsoft Edge, or Mozilla Firefox.

Navigate to the dashboard URL provided by the ICT department.

The dashboard loads automatically. No login credentials are required for the default deployment.

## 5.2  Layout & Navigation

| Zone | Description |
| --- | --- |
| Header Bar | Top - Ministry/ZINGSA branding, data source, last sync time, Refresh button |
| Left Navigation Rail | Icon-based vertical navigation for switching between modules |
| Inner Panel | Left side panel showing filtered asset list. Can be collapsed. |
| Map Zone | Central Leaflet map showing all surveyed assets as coloured markers |
| Right Analytics Panel | Right side panel showing charts, statistics, and selected record detail. |
| Footer Bar | System version, survey year, highway coverage, and record count |

## 5.3  Map View

Assets displayed as coloured pin markers. Colour indicates asset type.

Click a marker to select the survey record and view its details in the right panel.

Use the mouse wheel or track-pad to zoom in and out.

Use the Road Filter dropdown to filter markers by highway.

Figure: Web Dashboard Main Map View

## 5.4  Assets Panel

Click any record in the list to highlight it on the map and open its details.

Use the road filter at the top to narrow records by highway (A1-A5 and others).

## 5.5  Dashboard Module

Provides a high-level overview of survey statistics across all highways - total asset counts, condition distribution charts, and coverage summaries.

Figure: Web Dashboard Statistics and Charts Module

## 5.6  Highways Module

Per-highway breakdown of surveyed assets, showing asset count, condition ratings, and survey completion status for each of the surveyed highways: A1, A2, A3, A4, and A5.

## 5.7  Analytics Module

Asset condition distribution pie and bar charts

Road defect frequency analysis (cracks, potholes, rutting)

Survey activity timeline (records per date)

Passability analysis by road class

## 5.8  Survey Module

Structured tabular view of all survey records, filterable by asset type, road, surveyor, and date range.

## 5.9  Database Module

Raw data view of all survey records for searching, filtering, and individual record inspection.

## 5.10  Export Module

CSV - for use in Microsoft Excel or GIS software

JSON - raw data export for integration with other systems

PDF Report - formatted condition report for management briefings

## 5.11  Right-Side Analytics Panel

Asset type, road name, section, and GPS coordinates

Survey date and surveyor name

All condition fields specific to that asset type

Attached photo (if available)

Overall condition rating badge (Green=Good, Yellow=Fair, Red=Poor)

## 5.12  Refreshing Live Data

Phase 1 - Local Cache: On page load, locally cached data displayed instantly.

Phase 2 - Server Data: Server queried in background, merged with local records.

To force a live refresh from Central Database, click the Refresh Data button in the header.

# 6.  DATA SYNCHRONISATION & OFFLINE OPERATION

| Layer | Description | Behaviour |
| --- | --- | --- |
| Device Storage | Survey drafts stored in browser IndexedDB | Persistent across app restarts |
| Auto-Save | Form data saved to localStorage every 400ms | Crash recovery protection |
| Manual Sync | User-initiated batch upload to server | Via Sync All button in Queue tab |
| Server Cache | Server stores a local JSON cache of Central Database data | Reduces API call latency |
| Dashboard Cache | roads-data.json served as static fallback | Dashboard loads without server |
| Conflict Resolution | Server records take precedence; de-duplicated by record ID | Merging on Phase 2 load |

# 7.  SYSTEM ADMINISTRATION

7.1  Starting the Backend Server

Navigate to the backend/ directory on the server.

Run: python -m uvicorn main:app --reload --port 8000

API swagger documentation: http://[server-ip]:8000/docs

7.2  Environment Configuration

| Variable | Description | Example |
| --- | --- | --- |
| DATABASE_URL | Central Database API base URL | https://kf.Central Database.org/api/v2 |
| DATABASE_KEY | Central Database account username | motid_surveyor |
| DATABASE_PASSWORD | Central Database account password | [secured] |
| DATABASE_PROJECT_ID | Asset UID of the Roads survey form | a1b2c3d4e5... |
| OFFLINE_MODE | Serve only cached data without calling Central Database | false |

# 8.  TROUBLESHOOTING

| Symptom | Solution | Applies To |
| --- | --- | --- |
| GPS stuck at 'Waiting for signal' | Move to open area; ensure Location is ON in Android Settings; restart app. | Mobile App |
| App shows 'Offline' on WiFi | Check Server URL in Settings; tap 'Test Connection'; confirm backend is running. | Mobile App |
| Sync fails - Network Error | Verify internet; confirm backend server is running; retry after 30 seconds. | Mobile App |
| Lost draft after app crash | Open any form - yellow recovery banner appears - tap 'Restore Draft'. | Mobile App |
| Dashboard map is empty | Click 'Refresh Data' in header; verify backend is running and Central Database credentials are correct. | Web Dashboard |
| Data source shows 'Offline Cache' | Backend server may be down - contact ZINGSA ICT support. | Web Dashboard |
| Duplicate records on map | Click 'Refresh Data' - system auto-deduplicates on next load. | Web Dashboard |
| Export not downloading | Check browser download permissions; disable popup blocker for dashboard URL. | Web Dashboard |

# 9.  GLOSSARY

| Term | Definition |
| --- | --- |
| APK | Android Package Kit - the installation file format for Android applications |
| API | Application Programming Interface - rules for how software systems communicate |
| Capacitor | A cross-platform runtime that packages web apps into native Android/iOS apps |
| Culvert | A drainage structure allowing water to flow under a road |
| Drift | A concrete slab splashway for vehicles to cross a watercourse during low water |
| FastAPI | A high-performance Python web framework used to build the backend REST API |
| GIS | Geographic Information System - software for managing and analysing spatial data |
| GPS | Global Positioning System - satellite technology for geographic coordinates |
| HLDC | High Load Double Cross - a common precast concrete bridge beam type |
| Central Database | Online data collection platform used to store and manage survey submissions |
| Leaflet | An open-source JavaScript mapping library for the interactive map |
| MOTID | Ministry of Transport and Infrastructural Development |
| Next.js | A React-based web framework used to build the web dashboard |
| SADC | Southern African Development Community - standards body for road signs |
| Shelvet | A side drain channel (masonry or concrete) alongside the road pavement |
| Sync | Uploading locally stored survey records from the mobile device to the server |
| ZINGSA | Zimbabwe National Geospatial and Space Agency - the operator of this system |

# APPENDIX A - Central Database Integration

The backend server fetches survey data from the Central Database API. ZINGSA manages the Central Database organisation account and form assets.

Survey forms are designed in Central Database and deployed to the field via the mobile app.

The backend polls the Central Database API using the DATABASE_PROJECT_ID.

Data is cached server-side to reduce API load and support offline dashboard usage.

Central Database API endpoint: GET https://kf.Central Database.org/api/v2/assets/{ASSET_ID}/data.json

# APPENDIX B - API Endpoint Reference

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | / | Health check - returns server status and version |
| GET | /api/roads | Returns all road condition survey records |
| POST | /api/roads/submit | Accepts a new survey record submission from the mobile app |
| GET | /docs | Interactive Swagger UI API documentation |

--------------------------------------------------------------------------------

End of Document

Zimbabwe National Geospatial and Space Agency (ZINGSA)

In Partnership with: MOTID - Department of Roads

Document Version: v1.0  |  Issue Date: July 2026  |  Classification: INTERNAL USE ONLY